# Week 1 - Hand-Designed Control Logic

Forwarding unit, hazard-detection unit and branch resolution

CPEN 438 / CPEN 315 - Advanced Computer Architecture - University of Ghana, Legon

Project 3 - Hazard Watch

Project 3's Week 1 line reads: "Present papers; obtain team's seeded COCOBOD program; design forwarding-unit and hazard-detection-unit truth tables by hand before touching code."

This document is that deliverable. Nothing here is derived from the simulator — the simulator is written from this document in Week 2, and the Week 2 results are the check on whether this design was right.

## 1. The timing convention everything else depends on

Fix this first or nothing downstream is decidable.

The register file is written in the first half of a clock cycle and read in the second half (H&P Ch. 3 / App. C). A value written by an instruction in its WB stage during cycle t is therefore visible to any instruction reading the register file in ID during that same cycle t.

Number the stages of an instruction issued in slot p (no stalls):

| Stage | IF | ID | EX | MEM | WB |
| --- | --- | --- | --- | --- | --- |
| Cycle | p+0 | p+1 | p+2 | p+3 | p+4 |

Now derive the three facts the whole design rests on:

Fact A — distance-1 dependence needs EX/MEM forwarding. Producer at p finishes its ALU work at the end of cycle p+2; the result sits in the EX/MEM latch during cycle p+3. The consumer at p+1 needs it at the start of its EX, which is cycle p+3. The latch has it. Forward EX/MEM → ALU input.

Fact B — distance-2 dependence needs MEM/WB forwarding. Consumer at p+2 needs the value at cycle p+4. That is the producer's WB cycle. The consumer read the register file back in ID at cycle p+3 — before the write. So the register file did not supply it. The MEM/WB latch must. Forward MEM/WB → ALU input.

Fact C — distance-3 needs nothing. Consumer at p+3 reads the register file in ID at cycle p+4, the same cycle the producer writes it. Write-first-half / read-second-half means the read sees the new value. No forwarding path required.

This is why the Project-2 assembler pads to distance 3, and why exactly two forwarding paths — no more — are sufficient. If your team ever finds itself wanting a third path, the timing convention above is being violated somewhere.

## 2. Datapath with the added hardware

+--------------- HAZARD DETECTION UNIT ---------------+
                    |  freezes PC and IF/ID, injects a bubble into ID/EX  |
                    +----+------------------------------------------------+
                         | Stall
  +----+   +--------+    v    +--------+       +--------+       +--------+
  | PC |-->| IF/ID  |-------->| ID/EX  |------>| EX/MEM |------>| MEM/WB |
  +----+   +--------+         +--------+       +--------+       +--------+
     ^         ^ Flush             |                |                |
     |         |                   v                |                |
     |    +----+-----+      +-------------+         |                |
     |    |  BRANCH  |      |  ForwardA   |<--------+                |
     |    |  COMPARE |      |    MUX      |<-------------------------+
     |    |  (in ID) |      +------+------+         |                |
     |    +----+-----+             v                |                |
     |         |                +-----+             |                |
     +---------+                | ALU |             |                |
       PCSrc                    +-----+             |                |
                                   ^                |                |
                             +-----+-------+        |                |
                             |  ForwardB   |<-------+                |
                             |    MUX      |<------------------------+
                             +-------------+

   ForwardC / ForwardD muxes feed the ID-stage branch comparator from
   EX/MEM (not drawn; same shape as the ForwardA / ForwardB muxes).

Three new blocks, all combinational except the latch-control they drive:

| Block | Inputs | Outputs | Where it lives |
| --- | --- | --- | --- |
| Forwarding unit | EX/MEM.RegisterRd, EX/MEM.RegWrite, MEM/WB.RegisterRd, MEM/WB.RegWrite, ID/EX.RegisterRs, ID/EX.RegisterRt | ForwardA[1:0], ForwardB[1:0] | EX stage |
| Hazard-detection unit | ID/EX.MemRead, ID/EX.RegisterRt, IF/ID.RegisterRs, IF/ID.RegisterRt | Stall | ID stage |
| Branch resolution | ID-stage comparator + ForwardC, ForwardD | PCSrc, Flush | ID stage |

## 3. Forwarding unit — truth table

ForwardA selects the first ALU operand; ForwardB the second. Both are 2-bit.

| ForwardA | Source selected |
| --- | --- |
| 00 | Register file (ID/EX.RegisterRs value read in ID) |
| 10 | EX/MEM.ALUResult — the immediately preceding instruction |
| 01 | MEM/WB write-back value — two instructions back |

Truth table (rows evaluated top to bottom; the first matching row wins — this ordering is the priority rule):

| # | EX/MEM.RegWrite | EX/MEM.Rd ≠ 0 | EX/MEM.Rd = ID/EX.Rs | MEM/WB.RegWrite | MEM/WB.Rd ≠ 0 | MEM/WB.Rd = ID/EX.Rs | ForwardA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | × | × | × | 10 |
| 2 | 0 or match fails | — | — | 1 | 1 | 1 | 01 |
| 3 | otherwise |  |  |  |  |  | 00 |

ForwardB is the identical table with ID/EX.Rt substituted for ID/EX.Rs.

### As Boolean equations (what you actually wire in Logisim)

EXhitA  = EX/MEM.RegWrite  AND  (EX/MEM.Rd != 0)  AND  (EX/MEM.Rd = ID/EX.Rs)
WBhitA  = MEM/WB.RegWrite  AND  (MEM/WB.Rd != 0)  AND  (MEM/WB.Rd = ID/EX.Rs)

ForwardA[1] = EXhitA
ForwardA[0] = WBhitA  AND  EXhitA'          <- the priority term. Do not omit it.

### The priority rule, and why it is the graded trap

Row 1 must be checked before row 2. When two instructions in flight both write the same register, the one in EX/MEM is the newer one and its value is the architecturally correct one.

ADDI R1, R0, 111      <- older, will be in MEM/WB
ADDI R1, R0, 222      <- newer, will be in EX/MEM
ADD  R2, R1, R0       <- must read 222

Invert the priority and this returns 111 — silently, with no crash. It passes every single-dependence test. The instructor notes for this project say explicitly that a three-instruction dependency chain will be used to probe for exactly this at the defence. Test vector V6 in tests/hazard_test_vectors.py is that chain.

### What must NOT appear in this table

There is no row for "EX/MEM holds a load." A load's data does not exist until the end of MEM, so the EX/MEM latch holds only its computed address, not its data. Forwarding that address as if it were data is the error Project 3 section M warns about — it produces a plausible-looking wrong number rather than a crash. The hazard-detection unit exists precisely so this row can never be reached, and the simulator asserts on it.

## 4. Hazard-detection unit — truth table

Fires only for the load-use case.

| # | ID/EX.MemRead | ID/EX.Rt ≠ 0 | ID/EX.Rt = IF/ID.Rs and IF/ID uses Rs | ID/EX.Rt = IF/ID.Rt and IF/ID uses Rt | Stall |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | × | 1 |
| 2 | 1 | 1 | × | 1 | 1 |
| 3 | otherwise |  |  |  | 0 |

Stall = ID/EX.MemRead  AND  (ID/EX.Rt != 0)
         AND  [ (UsesRs  AND  (ID/EX.Rt = IF/ID.Rs)) + (UsesRt  AND  (ID/EX.Rt = IF/ID.Rt)) ]

### The UsesRs / UsesRt decode — the subtlety H&P's figure hides

H&P's textbook figure compares against IF/ID.RegisterRs and IF/ID.RegisterRt unconditionally. That is a simplification that produces false stalls on real instruction mixes, because not every instruction reads both fields:

| Instruction | Reads Rs? | Reads Rt? | Writes | Note |
| --- | --- | --- | --- | --- |
| ADD/SUB/AND/OR/SLT rd,rs,rt | yes | yes | rd |  |
| ADDI rt,rs,imm | yes | no | rt | rt is the destination, not a source |
| LW rt,imm(rs) | yes | no | rt |  |
| SW rt,imm(rs) | yes | yes | — | rt is the store data, a real source |
| BEQ/BNE rs,rt,L | yes | yes | — |  |
| NOP | no | no | — |  |

Two consequences your team must be able to state at the defence:

- ADDI after LW into the same rt is not a hazard — the ADDI overwrites that register, it does not read it. An HDU without the UsesRt term stalls here for nothing.

- SW after LW is a hazard on the store data. An HDU that only checks Rs misses it entirely and stores garbage. Test vector V7 covers this.

### What the stall physically does — three simultaneous actions

| Signal | Action | Effect |
| --- | --- | --- |
| PCWrite = 0 | PC holds its value | the same instruction is re-fetched |
| IF/ID.Write = 0 | IF/ID latch holds | the consumer stays in ID for another cycle |
| control-mux → 0 | all control signals into ID/EX zeroed | a bubble enters EX |

The bubble is what makes the stall cost exactly one cycle. One cycle later the load is in MEM/WB and the ordinary ForwardA/B = 01 path resolves the dependence normally — the load-use hazard is converted into a distance-2 hazard, not eliminated.

## 5. Branch resolution in ID

Moving the comparator into ID cuts the taken-branch penalty from three cycles to one, but creates a new class of data hazard, because ID is two stages earlier than EX.

Branch in ID at cycle t. What is where:

| Instruction | Stage at cycle t | Its result is… | Action |
| --- | --- | --- | --- |
| b−1 | EX | not computed yet | stall 1 cycle |
| b−2 | MEM (in EX/MEM) | in the EX/MEM latch | forward EX/MEM → ID (ForwardC/ForwardD) |
| b−2, and it is a LW | MEM | not available until end of MEM | stall 1 cycle |
| b−1, and it is a LW | EX | two cycles away | stall 2 cycles (one, then one more) |
| b−3 | WB | already written to the RF this cycle | nothing — read the RF |

BranchStall = IsBranch  AND 
   [ (ID/EX.RegWrite   AND  (ID/EX.Rd  != 0)  AND  (ID/EX.Rd  = Rs + ID/EX.Rd  = Rt))
   + (EX/MEM.MemRead   AND  (EX/MEM.Rd != 0)  AND  (EX/MEM.Rd = Rs + EX/MEM.Rd = Rt)) ]

ForwardC = EX/MEM.RegWrite  AND  (EX/MEM.Rd != 0)  AND  (EX/MEM.Rd = IF/ID.Rs)  AND  BranchStall'
ForwardD = EX/MEM.RegWrite  AND  (EX/MEM.Rd != 0)  AND  (EX/MEM.Rd = IF/ID.Rt)  AND  BranchStall'

Note the two-stall case needs no special logic: after the first stall the load has advanced from EX to MEM, the second condition now matches, and the unit stalls again. The correct behaviour falls out of re-evaluating the same combinational logic each cycle.

Flush. When the comparator says taken, the instruction fetched in the same cycle is on the wrong path. Clear IF/ID (IF.Flush = 1) and load the PC with the target. Cost: exactly one cycle. When the comparator says not taken, do nothing — the sequential instruction already in IF/ID is correct. A pipeline that flushes unconditionally passes a taken-branch test and fails a not-taken one; vectors V4 and V5 are that pair.

## 6. Hazard classification scheme for the report

Project 3 section L requires a per-hazard classification table. These are the categories, fixed now so Week 2's instrumentation emits them directly:

| Class | Trigger | Cost | Resolved by |
| --- | --- | --- | --- |
| RAW forwarded EX/MEM→EX | distance-1 ALU dependence | 0 | ForwardA/B = 10 |
| RAW forwarded MEM/WB→EX | distance-2 dependence | 0 | ForwardA/B = 01 |
| RAW forwarded EX/MEM→ID | branch operand from b−2 | 0 | ForwardC/D |
| LOAD-USE stall required | HDU fires | 1 | bubble, then MEM/WB forward |
| BRANCH-DATA stall required | branch operand not ready | 1 or 2 | bubble, then EX/MEM→ID forward |
| CONTROL flush (taken branch) | comparator says taken | 1 | IF/ID cleared |
| no hazard | independent instruction | 0 | — |

## 7. Hand traces to carry into Week 2

Do these on paper before running anything. They are the acceptance criteria.

### Trace 1 — the worked example from Project 3 section F

ADD R1, R2, R3
SUB R4, R1, R5      <- RAW dist-1 on R1
AND R6, R4, R1      <- RAW dist-1 on R4, dist-2 on R1
OR  R7, R6, R1      <- RAW dist-1 on R6, dist-3 on R1 (register file)

| Cycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADD | IF | ID | EX | MEM | WB |  |  |  |
| SUB |  | IF | ID | EX | MEM | WB |  |  |
| AND |  |  | IF | ID | EX | MEM | WB |  |
| OR |  |  |  | IF | ID | EX | MEM | WB |
| ForwardA |  |  |  | 00 | 10 | 10 | 10 |  |
| ForwardB |  |  |  | 00 | 00 | 01 | 00 |  |

Four instructions, zero stalls, 8 cycles, CPI = 2.00 (dominated by fill; the steady-state contribution is 0 stalls/instruction). Note AND's ForwardB = 01 — that is Fact B in action — and OR's ForwardB = 00, which is Fact C.

### Trace 2 — replace the first instruction with a load

LW  R1, 0(R2)
SUB R4, R1, R5      <- LOAD-USE
AND R6, R4, R1
OR  R7, R6, R1

| Cycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LW | IF | ID | EX | MEM | WB |  |  |  |  |
| SUB |  | IF | ID | ID | EX | MEM | WB |  |  |
| AND |  |  | IF | IF | ID | EX | MEM | WB |  |
| OR |  |  |  |  | IF | ID | EX | MEM | WB |
| Stall |  |  |  | 1 |  |  |  |  |  |
| bubble in EX |  |  |  | ✓ |  |  |  |  |  |

Exactly one stall. 9 cycles instead of 8. Stall contribution to CPI = 1 stall / 4 instructions = 0.25 — the precise number Project 3 section F says the report must show cycle by cycle.

### Trace 3 — the load-use bubble is not a NOP the assembler inserted

Worth stating explicitly in the report, because the two look identical in a pipeline diagram and are architecturally different things: the Project-2 NOP occupies a slot in the instruction memory and is fetched, decoded and retired like any instruction. The Project-3 bubble is created by the hazard-detection unit inside the pipeline, occupies no instruction-memory slot, and never retires. This distinction is what makes the two CPI figures in the Week 2 results non-comparable, and is discussed there.

## 8. Week 1 checklist

- ☒ Timing convention fixed and justified (§1)

- ☒ Datapath extension diagram (§2)

- ☒ Forwarding-unit truth table + Boolean equations + priority rule (§3)

- ☒ Hazard-detection-unit truth table + UsesRs/UsesRt decode (§4)

- ☒ Branch resolution + flush logic (§5)

- ☒ Hazard classification scheme fixed (§6)

- ☒ Three hand traces with expected cycle counts (§7)

- ☐ Paper-review presentation delivered — see week1_paper_review_slides.md

- ☐ Project proposal submitted — see week1_proposal.md

- ☐ Team seed received from instructor and recorded on the proposal

- ☐ Repository initialised, AI-use declaration started