# Week 1 - Project Proposal

Proposal, requirements specification and AI-use declaration

CPEN 438 - Advanced Computer Architecture - University of Ghana, Legon

GROUP 5

Project 3 - Hazard Watch

| Field | Entry |
| --- | --- |
| Project number & title | 3 , Hazard Watch: Forwarding, Stalling and Branch Resolution in a COCOBOD Yield-Estimation Datapath |
| Team members & roles (Part I §9) | Hardware Lead: Nyavor Cyril- 11023595, owns the forwarding-unit and hazard-detection-unit Logisim subcircuits; must trace a forwarding decision live at the defence. C/C++ Lead: Prince Philips- 11218951 , owns the golden simulator's hazard logic; must explain why a given hazard was or was not forwardable. Test/Docs Lead: Samia Soleimani- 11010910, owns the hand-derived hazard test vectors and the hazard-classification table. Project Manager: Ishaan Bhardwaj- 11004272, repo, integration branch, weekly reports, defence chairing. Python/MATLAB Lead: Ishaan Bhardwaj ,owns gen_cocobod_instrs.py and the Week 3 stall_cpi_model.m. |
| Assigned seed | Not yet issued at the time of this submission. Part V §5 assigns the seed at project allocation; ours had not been received when the Weeks 1–2 deliverables were due. Rather than submit results generated from an undocumented number, we are using a declared provisional seed. Provisional seed: 16993 Derivation. Drawn once on 19 August 2026 from the operating system's cryptographic entropy source and recorded here verbatim; it has not been re-drawn or selected from several candidates: python3 -c "import random; print(random.SystemRandom().randrange(10000, 100000))" Because the program generator is deterministic given the seed, recording the drawn value is sufficient for full reproducibility — the draw itself does not need to be repeatable. This satisfies the property the integrity framework actually depends on — that no two teams' instances coincide — while being fully reproducible by the marker (the generator is deterministic given the seed) and visibly labelled as unofficial. On receipt of the official seed, python3 rebuild.py --seed <official> --status official regenerates the program, reruns the simulator and test vectors, and rewrites every number in the Week 2 report in a single command. |
| Chosen papers for review | Smith & Sohi, "The Microarchitecture of Superscalar Processors," Proc. IEEE 83(12), 1995, 1609–1624, DOI 10.1109/5.476078. Patterson & Séquin, "RISC I: A Reduced Instruction Set VLSI Computer," ISCA '81, 443–457, DOI 10.1145/285930.285981. |
| Problem statement (team's own restatement) | The Project-2 GhanaCore-5 pipeline is correct only because its assembler pads every dependence with NOPs , a software fix for a hardware problem that inflates code size and cycle count in direct proportion to how dependence-dense the program is. Our COCOBOD regional yield-estimation routine is deliberately dependence-dense, so it pays that cost heavily. We will add three pieces of hardware which include, a forwarding unit, a hazard-detection unit, and an ID-stage branch comparator with flush logic that resolve those dependences in hardware at run time, and we will quantify exactly how much each one buys. Specifically we will establish which hazards forwarding can eliminate entirely, which it provably cannot, and what the residual cost of the ones it cannot is. |
| Hardware/software configuration | Logisim Evolution ,GCC ; Python 3.12; MATLAB (Week 3, CPI model). Golden simulator |
| Planned innovation-challenge idea (revisable by Week 3) | A forwarding-aware instruction scheduler that reorders the COCOBOD routine to convert load-use hazards into forwardable RAW hazards wherever legal, i.e. move an independent instruction into the slot immediately after each load. Evaluated by stall count before versus after, with a proof obligation that the reordering preserves program semantics (no register redefined between the moved instruction's producers and consumers). Our current program has exactly one load-use hazard and one obviously-movable independent instruction, so we expect a measurable, if small, result and a clean argument. |
| Risk/blocker anticipated | Primary risk: we are entering Project 3 without a Project 2 artefact. Project 3 assumes a working five-stage datapath to extend. Mitigation: the brief permits starting from the instructor's reference pipeline, and we have additionally built our own five-stage golden simulator from scratch (src/hazard_pipeline_sim.c) whose --mode baseline configuration is the Project-2 behaviour — no forwarding, no interlock, assembler-inserted NOPs. This gives us a legitimate, self-consistent baseline to measure against in Week 3 without depending on work we do not have. Secondary risk: the official seed had not been issued when Weeks 1 and 2 were submitted. Mitigation: the provisional-seed scheme above, declared rather than concealed, plus a single-command regeneration path. Our stated conclusions are structural (which hazard classes forwarding resolves, and at what cost each) and do not depend on the seed; only the exact cycle counts do, and those are regenerated automatically. Tertiary risk: the Logisim port in Week 3 is where teams historically lose time. Mitigation: build and verify the forwarding unit and the HDU as isolated subcircuits against the test vectors before wiring them into the datapath, per the Project 2 instructor note. |
| Repository URL | https://github.com/Ier2077/CPEN-438-Group-5 |

## Requirements specification (Week 1 deliverable, Part I §4)

### Functional

| ID | Requirement | Verified by |
| --- | --- | --- |
| F1 | The Logisim circuit and the golden simulator execute the assigned COCOBOD routine with forwarding enabled and zero manual NOPs, producing identical state at every cycle. | Week 3 cross-validation |
| F2 | Every RAW dependence resolvable by forwarding is resolved with zero stall cycles. | V1, V2 |
| F3 | Every load-use hazard costs exactly one stall cycle. | V3, V7 |
| F4 | Branches resolve in ID; a taken branch costs exactly one flush cycle, a not-taken branch costs zero. | V4, V5 |
| F5 | Forwarding priority selects the most recent producer. | V6 |
| F6 | A load result is never forwarded from EX/MEM. | negative test (--no-hdu) |

### Non-functional

| ID | Requirement |
| --- | --- |
| N1 | Forwarding-mux selection is computed combinationally within the same cycle — the forwarding paths add no pipeline latency. |
| N2 | The simulator emits a machine-readable per-cycle trace (trace_*.csv) whose columns correspond one-to-one with Logisim wires, so Week 3 cross-validation can be automated rather than eyeballed. |
| N3 | Every result is reproducible from the seed alone: gen_cocobod_instrs.py --seed S → hazard_sim --compare. |

### Inputs / outputs

- Input: the team's uniquely seeded COCOBOD instruction sequence, generated by gen_cocobod_instrs.py, engineered to contain several forwardable RAW hazards at distance 1 and 2, exactly one load-use hazard, and exactly one conditional branch.

- Output: per-cycle forwarding-mux selection trace; hazard-classification table (forwarded / stalled / flushed) with the cycle cost of each; CPI comparison against the NOP-padded baseline.

## AI-use declaration — starting entry (Part I §7)

| Date | Component | Assistance used | What we verified ourselves |
| --- | --- | --- | --- |
| 15/8/26 | docs/week1_design_truth_tables.md | AI assistance used to understand the truth tables and hand traces. | Every row of both truth tables re-derived by hand from the stage-timing table in §1 before acceptance; the three hand traces in §7 were worked on paper independently and matched. |
| 15/8/26 | src/hazard_pipeline_sim.c | AI assistance used to draft the simulator. | Cross-validated against the eight hand-derived test vectors, whose expected values were written before the code was run. Each team member must be able to explain the forwarding block and the HDU block line by line. |
| 15/8/26 | src/gen_cocobod_instrs.py report | AI assistance used to draft the generator. | Generated program inspected instruction by instruction against the intended hazard inventory. |
| 19/8/26 | REPORT | polish report using AI | Compiles results into a report and used AI to provide finishing touches |