# Weekly Course Project Report - Week 2

Golden simulator, hazard test vectors and CPI results

CPEN 438 - Advanced Computer Architecture - University of Ghana, Legon

Group 5

Project 3 - Hazard Watch

## 1. Weekly Objective and Progress Summary

Main objective for the week: Implement the forwarding unit and hazard-detection unit in the golden C simulator and verify them against the test vectors hand-derived in Week 1. Because the team holds no Project 2 artefact, an added objective was to build the five-stage baseline datapath the hazard hardware attaches to.

Overall project completion: 45%

Summary of progress achieved:

The golden simulator is complete and runs in two configurations selected at run time. --mode baseline reproduces Project-2 semantics — no forwarding, no interlock, correctness guaranteed by assembler NOP padding. --mode hazard adds the three blocks this project is about: EX/MEM and MEM/WB forwarding to the ALU inputs, a hazard-detection unit for the load-use case, and branch resolution in ID with a one-cycle flush and its own forwarding path to the comparator.

All eight hand-derived test vectors pass, including the negative test that disables the hazard-detection unit and confirms a load result is never forwarded from EX/MEM.

On the assigned COCOBOD routine the result is 47 cycles reduced to 25, a 1.88× speedup, with the final architectural state identical between the two configurations across all registers and all data memory. Sixteen of eighteen true dependences are resolved at zero cost; the two that cost a cycle each are the load-use hazard and one branch-operand hazard — both predicted in the Week 1 design document before any code existed.

The most instructive result was not the speedup. The baseline's CPI-per-retired-instruction is 1.09 against the hazard configuration's 1.32 — the baseline looks better on that metric while running nearly twice as slow, because 24 of its 43 instructions are NOPs that retire one per cycle and dilute the average. Reporting the wrong one of those two figures inverts the conclusion. This is discussed in section 4.

## 2. Meetings Held

Replace dates, times and attendance with what actually occurred before signing.

| Date | Duration/Mode | Members Present | Main Agenda | Decisions and Outcomes |
| --- | --- | --- | --- | --- |
| [18/8/26] | 1hr,class | all | Implementation sync: Philips walked the team through the simulator; Cyril checked it against the Week 1 truth tables | Two corrections agreed. (i) The priority term in the ForwardA equation was initially implicit in row ordering; made explicit as a Boolean product so the Logisim wiring in Week 3 is unambiguous. (ii) Decided that a mis-forward of a load result must abort loudly rather than warn, so the failure mode cannot be missed. |
| [19/8/26] | 20min , online | all | Verification review: Samia presented the vector results; team agreed the reporting metric; Week 3 planned | Agreed to report both CPI figures with an explicit argument for which one answers the question, rather than quietly selecting the flattering one. Agreed that the Logisim port in Week 3 will build and verify each unit as an isolated subcircuit before wiring, per the Project 2 instructor note. Identified that our seeded branch resolves not-taken, so a taken-branch timing diagram must come from test vector V4. |

Technical alternatives considered. For the load-use case we considered suppressing the forwarding-unit output during the stall instead of injecting a bubble. Rejected: a bubble is what real hardware does and what the Logisim implementation will need, and the alternative would have made the software and hardware diverge in the one place cross-validation matters most.

Unresolved issues requiring further investigation. None outstanding from this week. Carried forward: the branch-direction gap noted above, and the choice of Level-3 extension.

## 3. Work Completed and Individual Contributions

| Member | Assigned Task | Work Completed | Evidence/Output | Status |
| --- | --- | --- | --- | --- |
| Ishaan | Integration, full-program comparison, CPI analysis | Merged the implementation branches; ran the full baseline-versus-hazard comparison; produced the CPI analysis and the argument for which metric is comparable; wrote the automated regeneration script | results/run_seed16993.txt; rebuild.py; §4 of this report | Completed |
| Cyril | Design-conformance review | Mapped every row of both truth tables to the line of C implementing it; identified that the forwarding priority term was implicit rather than explicit and had it rewritten; documented the simultaneous dual-forward case at cycle 14 | Conformance note; commit history | Completed |
| Philips | Golden simulator implementation | Implemented the five-stage datapath, the forwarding unit with correct priority, the hazard-detection unit with per-opcode source decode, ID-stage branch resolution with flush, and the assembler NOP padder for the baseline configuration | src/hazard_pipeline_sim.c (~640 lines); all vectors passing | Completed |
| Samia | Verification and reproducibility | Ran all eight vectors against the implementation; produced the hazard-classification table; verified cross-validation of architectural state; wrote the README reproduction commands | tests/hazard_test_vectors.py; results/hazard_classification.csv; results/test_vectors.txt; README.md | Completed |

## 4. Work Process and Technical Activities

Methods, tools, and procedures used. The simulator is C, built with gcc -O2 -Wall -Wextra and warning-clean. It is cycle-driven: each cycle computes the next set of pipeline latches from the current set, with the write-back stage writing the register file before the decode stage reads it, which models the write-first-half / read-second-half convention fixed in Week 1. The forwarding unit, the hazard-detection unit and the branch-comparator forwarding are separate blocks in the source, positioned in the stages they physically occupy, so that the Week 3 Logisim port is a transcription rather than a redesign. Trace output is CSV with columns named after the intended Logisim wires — ForwardA, ForwardB, ForwardC, ForwardD, Stall, Flush — so Week 3 can be cross-validated signal by signal instead of by eye.

Testing, verification, or review conducted.

Eight vectors, every expected value derived by hand in Week 1 before the code was run:

| Vector | What it pins | Expected | Result |
| --- | --- | --- | --- |
| V1 | Distance-1 RAW, EX/MEM forward | 8 cycles, 0 stalls | Pass |
| V2 | Distance-2 RAW, MEM/WB forward | 7 cycles, 0 stalls | Pass |
| V3 | Load-use | 8 cycles, exactly 1 stall | Pass |
| V4 | Branch taken | 10 cycles, 1 stall, 1 flush, skipped instruction never writes | Pass |
| V5 | Branch not taken | 10 cycles, 1 stall, 0 flushes | Pass |
| V6 | Forwarding priority on a 3-instruction chain | Consumer reads 222, not the stale 111 | Pass |
| V7 | SW reads rt as a source | 8 cycles, 1 stall | Pass |
| Negative | Hazard-detection unit disabled | Load mis-forward detected and aborted | Pass |

V4 and V5 are a deliberate pair: a pipeline that flushes unconditionally passes V4 and fails V5. V6 is the three-instruction dependency chain the project's instructor notes say will be probed at the defence — inverting the forwarding priority passes V1 and V2 and silently returns a stale value here.

Full-program verification: baseline and hazard configurations produce identical final state across all 32 registers and all 256 data-memory words. This is what makes the reconstructed baseline a legitimate measurement reference rather than a convenient assumption — if the two disagreed anywhere, one of them would be wrong.

Results obtained.

| Metric | Baseline (Project 2) | Forwarding + HDU |
| --- | --- | --- |
| Total clock cycles | 47 | 25 |
| Instructions in memory | 43 (24 NOPs) | 19 |
| Load-use stall cycles | 0 | 1 |
| Branch-data stall cycles | 0 | 1 |
| CPI (useful instructions) | 2.4737 | 1.3158 |
| CPI (all retired instructions) | 1.0930 | 1.3158 |
| Speedup | — | 1.88× |
| Cross-validation | identical, 0 mismatching words |  |

Forwarding activity: 10 activations via EX/MEM to the ALU, 5 via MEM/WB, 1 via EX/MEM to the branch comparator.

Interpretation of the two CPI figures. Both are arithmetically correct and they point in opposite directions. CPI is only comparable across designs when the instruction count is held constant, and the whole intervention here changes the instruction count — it removes 24 NOPs. The baseline's low CPI is an artefact of those NOPs retiring one per cycle. The comparable metrics are total cycle count for identical program semantics (47 against 25) and CPI normalised to useful instructions (2.47 against 1.32). We report both and state which answers the question.

Interpretation of the load-use stall. At the stall cycle the load is in EX and its data does not exist until the end of MEM; IF/ID freezes, the PC holds, and a bubble enters EX. Two cycles later the ordinary MEM/WB forwarding path supplies the value. The stall therefore does not eliminate the hazard — it converts a load-use hazard into a distance-2 hazard that forwarding can handle. That is why the cost is exactly one cycle, never zero and never two.

## 5. Challenges, Changes, and Corrective Actions

| Challenge or Deviation | Effect on Project | Action Taken/Proposed | Responsible Member |
| --- | --- | --- | --- |
| Integrity seed still not issued | All reported figures depend on a program generated from a provisional seed | Provisional seed 16993 in use and declared with its derivation. rebuild.py regenerates the program, reruns the simulator and test vectors, and rewrites every number in this report from the official seed in one command | ISHAAN |
| No Project 2 artefact to extend | No baseline datapath existed | Five-stage simulator built with a --mode baseline configuration reproducing Project-2 semantics, cross-validated against the hazard configuration. Declared as our reconstruction, not as a Project-2 result | PHILIPS |
| First hazard-detection draft compared IF/ID.Rt unconditionally | Produced a false stall whenever ADDI followed LW into the same register | Per-opcode UsesRs/UsesRt decode added. Vector V7 added to pin the SW-after-LW case, which the naive version got wrong in the opposite direction by missing a real hazard on the store data | PHILIPS, SAMIA |
| Forwarding priority term was implicit in row ordering | Correct in C, ambiguous for the Week 3 Logisim wiring | Rewritten as an explicit Boolean product, ForwardA[0] = WBhitA · EXhitA' | CYRIL |
| The seeded program's branch resolves not-taken, so flush count is zero | Project 3 §L requires timing-diagram evidence for both branch directions | Test vector V4 provides a taken branch and will be used for the Week 3 timing diagram | SAMIA |

Support required from the lecturer: the assigned integrity seed.

## 6. Plan for the Next Week

| Planned Task or Deliverable | Responsible Member(s) | Deadline | Expected Evidence |
| --- | --- | --- | --- |
| Build the forwarding unit and hazard-detection unit as isolated Logisim subcircuits and verify each independently | CYRIL | 19/8/26 | .circ file, per-unit test results |
| Wire the verified subcircuits into the datapath | CYRIL, [M3] | 19/8/26 | Full circuit |
| Cross-validate the full COCOBOD routine against trace_hazard.csv, cycle by cycle | SAMIA | 19/8/26 | Signal-level comparison log |
| Capture mux-select timing diagrams for one branch-taken and one branch-not-taken execution | SAMIA | 19/8/26 | Annotated screenshots |
| MATLAB analytical CPI model as a function of hazard density, validated against measured stall counts | [M1] | 19/8/26 | stall_cpi_model.m, plot |
| Begin the innovation component: forwarding-aware instruction scheduler | [M1], [M3] | 19/8/26 | Prototype, stall counts before and after |
| Week 3 report and individual reflections | All | [date] | This form, completed |

Critical decision, resource, or lecturer support required: the assigned integrity seed. Confirmation of which Level-3 extension to pursue — our current preference is the forwarding-aware scheduler described in Project 3 §N, since our routine contains exactly one load-use hazard and one legally movable independent instruction.

## 7. Team Accountability Declaration

We confirm that this report accurately represents the meetings, work completed, individual contributions, challenges, and evidence produced during the reporting week.

Team Leader: ISHAAN BHARDWAJ Date: 19/8/26

Team Members' Initials: S.S , P.P , N.C

Evidence repository/link: https://github.com/Ier2077/CPEN-438-Group-5