University of Ghana, Legon

Department of Computer Engineering

University of Ghana, Legon

Department of Computer Engineering

CPEN 438: ADVANCED COMPUTER ARCHITECUTRE AND DESIGN

# Project 3 :Hazard Watch: Forwarding, Stalling and Branch Resolution in a COCOBOD Yield-Estimation Datapath

GROUP 5

Ishaan Bhardwaj (11004272)

Nyavor Cyril (11023595)

Prince Philips (11218951)

Samia Soleimani (11010910)

Contents

Project 3 :Hazard Watch: Forwarding, Stalling and Branch Resolution in a COCOBOD Yield-Estimation Datapath	1

Abstract	2

1. Introduction	2

2. Problem Statement	2

3. Objectives and Outcomes	2

4. Background and Related Work	3

5. Architecture and Design	3

5.1 Timing convention	3

5.2 Forwarding unit	3

5.3 Hazard-detection unit	3

5.4 Branch resolution	4

6. Implementation	4

7. Experimental Setup	4

8. Generating a Hazard-Density Sweep	4

9. Results	5

9.1 Assigned routine	5

9.2 Hazard resolution	5

9.3 Density suite	5

9.4 Two independent simulators	6

9.5 Logisim hazard units — and a bug found by cross-validation	6

9.6 Negative test: what the hazard unit prevents	7

9.7 Taken-branch case	7

10. Verification and Validation	7

11. Which CPI, and Why It Matters	7

12. Analytical CPI Model	8

13. Innovation Component: Forwarding-Aware Scheduling	8

13.1 Motivation	8

13.2 Method	8

13.3 Results	8

14. Limitations and Threats to Validity	9

15. Conclusions	9

16. Reproducibility Statement	9

17. Individual Contributions	10

18. AI-Use Summary	10

19. Acknowledgements	10

20. References	10

Appendices	10

## Abstract

A five-stage pipelined datapath executing a regional cocoa-yield estimation routine spends most of its time waiting. Under the Project 2 baseline — no forwarding, hazards resolved by NOP padding at assembly time — a 19-instruction routine expands to a 43-instruction image and requires 47 clock cycles, a CPI of 2.474 over useful instructions. We implement full EX/MEM and MEM/WB operand forwarding, a load-use interlock with per-opcode source decoding, and branch resolution advanced to the decode stage. The routine then completes in 25 cycles, CPI 1.316, a speedup of 1.880×, with architectural state identical to the baseline. Correctness is established three ways: ten hand-derived test vectors whose expected values were written before implementation, two independently written simulators agreeing cycle for cycle across eleven trace columns in both configurations, and three Logisim subcircuits verified against the Week 1 truth tables by 3,000 random vectors each and then against the golden simulator on every cycle of the assigned routine. Two stall cycles survive forwarding; we show both are consequences of instruction ordering rather than hardware limits, and a forwarding-aware scheduler removes them across an eleven-program density suite, cutting 620 cycles to 580 with semantic equivalence verified on every program.

## 1. Introduction

Pipelining raises throughput by overlapping instruction execution, but the overlap is only safe when each instruction’s operands are available when it needs them. Where they are not, the pipeline has a hazard, and the architecture must either stall until the value arrives or route it from wherever it currently exists. Neither choice is free: stalling costs cycles, forwarding costs wires, multiplexers and control logic.

This project quantifies that trade-off on a concrete workload. The Ghana Cocoa Board (COCOBOD) estimates regional cocoa yield from predictor variables — rainfall, fertiliser application, tree age — combined under weights and clamped against a regional ceiling. The computation is short, arithmetic-dense and heavily dependent: nineteen instructions carry nineteen dependence events. That makes it an unusually clear specimen, because the hazards are not incidental to the program, they are the program.

## 2. Problem Statement

- What does hazard-handling hardware buy, in cycles, on a real workload rather than a synthetic one?

- Which stalls are irreducible consequences of pipeline structure, and which are artefacts of instruction ordering?

- Can the hazard-control logic be shown correct by more than one independent route?

## 3. Objectives and Outcomes

| # | Objective | Outcome |
| --- | --- | --- |
| O1 | Cycle-accurate simulator in both configurations | Met |
| O2 | Derive forwarding and hazard logic from first principles, not from a reference figure | Met — §5 |
| O3 | Measure CPI in both configurations, semantics held constant | Met — 47→25 cycles |
| O4 | Reproduce the logic in Logisim and cross-validate cycle by cycle | Partially met — the three hazard units are built and verified against the simulator on every cycle; they are not wired into a full datapath. §9.5 and §14 |
| O5 | Analytical CPI model validated against measurement | Met |
| O6 | Innovation: reduce surviving stalls without hardware change | Met — §13 |

## 4. Background and Related Work

Hennessy and Patterson give the canonical treatment of pipeline hazards and forwarding in Appendix C, and the CPI-comparability argument this report relies on in §11 in Chapter 1: CPI is a meaningful basis for comparison only when instruction count is held constant. Our baseline violates that condition by construction, and §11 shows what happens if the caution is ignored.

## 5. Architecture and Design

### 5.1 Timing convention

Three facts about when values exist, established before any logic was drawn.

Fact A. An ALU result is computed in EX and latched into EX/MEM at the end of that cycle, so it is available to a following instruction’s EX one cycle later. Fact B. A load’s data is not available until the end of MEM. Fact C. The register file is written in the first half of a cycle and read in the second, so a producer in WB and a consumer in ID in the same cycle need no forwarding.

The required paths follow as a conclusion: distance-1 needs EX/MEM → EX; distance-2 needs MEM/WB → EX; distance-3 needs nothing. Exactly two forwarding paths into EX are sufficient, and a third would be dead logic. Rows 7 and 18 of the classification table in §9.2 are distance-3 dependences classified NONE, which is Fact C observed rather than assumed.

### 5.2 Forwarding unit

EXhitA  = EX/MEM.RegWrite AND (EX/MEM.Rd ≠ 0) AND (EX/MEM.Rd = ID/EX.Rs)
WBhitA  = MEM/WB.RegWrite AND (MEM/WB.Rd ≠ 0) AND (MEM/WB.Rd = ID/EX.Rs)

ForwardA[1] = EXhitA
ForwardA[0] = WBhitA AND (NOT EXhitA)

ForwardB is identical with ID/EX.Rt. The Rd ≠ 0 guard exists because R0 is hardwired to zero, so an instruction nominally writing it must never be a forwarding source. The NOT EXhitA term encodes producer priority: when both stages hold a pending write to the same register, EX/MEM is the more recent and must win. Reversed, it passes every single-dependence test and fails only on back-to-back dependent instructions — vector V6 exists specifically to catch it. A consequence worth noting is that the encoding 11 is unreachable by construction.

### 5.3 Hazard-detection unit

LoadUseStall = ID/EX.MemRead AND (ID/EX.Rt ≠ 0)
               AND [ (UsesRs AND (ID/EX.Rt = IF/ID.Rs))
                  OR (UsesRt AND (ID/EX.Rt = IF/ID.Rt)) ]

The UsesRs/UsesRt terms are decoded per opcode and are where this design departs from the textbook figure, which compares Rt unconditionally. That is wrong in both directions on this instruction subset. ADDI Rt, Rs, imm writes Rt and never reads it, so an unconditional comparison stalls for a dependence that does not exist — vector V8. SW Rt, imm(Rs) reads Rt as store data, so a unit ignoring Rt misses a genuine load-use dependence — vector V7.

We did not take this from the literature. V7 was hand-written before any code existed, failed on first execution, and tracing it by hand located the fault in the detection condition rather than in the vector.

Vector V9 pins the other boundary: the load-use stall is a distance-1 phenomenon only. One independent instruction between load and consumer is enough, because by the consumer’s EX the load sits in MEM/WB and ForwardA = 01 supplies it. A unit that fires on any load-then-use pair regardless of distance passes V7 and silently inflates CPI everywhere else.

### 5.4 Branch resolution

Resolving branches in ID rather than MEM cuts the taken-branch penalty from three cycles to one, but creates a new hazard class, because a branch in ID reads registers a producer may not yet have written.

BranchStall = IsBranch AND
    [ ( ID/EX.RegWrite AND (ID/EX.Rd ≠ 0) AND (ID/EX.Rd matches Rs or Rt) )
   OR ( EX/MEM.MemRead AND (EX/MEM.Rd ≠ 0) AND (EX/MEM.Rd matches Rs or Rt) ) ]

ForwardC = EX/MEM.RegWrite AND (EX/MEM.Rd ≠ 0)
           AND (EX/MEM.Rd = IF/ID.Rs) AND (NOT BranchStall)
ForwardD = ... identically with IF/ID.Rt ...

ForwardC and ForwardD are single bits because, by Fact C, there is only one useful forwarding source into ID. The two-stall case needs no extra logic: after the first stall the load has advanced from EX to MEM, the second disjunct matches, and the unit stalls again.

## 6. Implementation

| Component | Language | Author |
| --- | --- | --- |
| sim/hazard_pipeline.c, pipeline_core.h, asm_core.h | C | Prince Philips |
| WEEK3_SUBMISSION/.../hazard_pipeline_sim.c (golden, Week 2) | C | Prince Philips |
| tools/gen_cocobod_instrs.py, gen_density_suite.py, sweep.py, plot_cpi.py | Python | Ishaan Bhardwaj |
| matlab/stall_cpi_model.m | MATLAB | Ishaan Bhardwaj |
| logisim/GhanaCore5_HazardUnits.circ, tools/gen_logisim_roms.py, circ_patch.py, logisim_check.py | Logisim / Python | Nyavor Cyril |
| tests/hazard_test_vectors.py, extra_test_vectors.py, tools/crossval.py, tools/scheduler.py | Python | Samia Soleimani |

Two simulators were written independently: the Week 2 golden simulator and a second implementation in sim/. Both execute the same programs from source, and §10 reports their agreement.

## 7. Experimental Setup

Workload. The assigned COCOBOD routine (19 useful instructions, 43 after padding) plus an eleven-program density suite of 48 useful instructions each, spanning target hazard densities 0.0 to 1.0.

Configurations. Baseline (NOP-padded, no forwarding); forwarding + hazard detection; forwarding + scheduling.

Controlled variable. Instruction subset and program semantics identical across configurations, verified rather than assumed: architectural state is compared word by word after every run, and states_match = 1 on all eleven suite programs.

## 8. Generating a Hazard-Density Sweep

Re-running the Week 1 generator with different seeds does not produce a density sweep. That generator builds one fixed program shape; the seed permutes register allocation, selects data constants and swaps one independent pair, leaving the dependence structure invariant. Several seeds would yield one point on the plot measured several times, and any curve through it would be an artefact.

gen_density_suite.py varies the structure directly, holding domain and instruction subset fixed and changing only how often an instruction consumes its immediate predecessor’s result. Eleven programs, 48 useful instructions each, target density 0.0 to 1.0 in steps of 0.1. The team seed is mixed into every variant.

## 9. Results

### 9.1 Assigned routine

| Metric | Baseline | Forwarding |
| --- | --- | --- |
| Hardware | no forwarding, no HDU | forwarding + HDU |
| Instructions in the image | 43 | 19 |
| of which NOPs | 24 | 0 |
| Total cycles | 47 | 25 |
| Load-use stall cycles | 0 | 1 |
| Branch-data stall cycles | 0 | 1 |
| Flush cycles | 0 | 0 |
| CPI over useful instructions | 2.4737 | 1.3158 |

Speedup 1.880× — 22 cycles saved, 46.8%. The baseline inserts 24 NOPs into a 19-instruction program: more than half of everything it executes is padding.

### 9.2 Hazard resolution

Nineteen dependence events. Seventeen resolve at zero cost:

| Resolution | Count |
| --- | --- |
| ForwardA = 10 (EX/MEM → ALU) | 7 |
| ForwardA = 01 (MEM/WB → ALU) | 5 |
| ForwardB = 10 (EX/MEM → ALU) | 2 |
| ForwardB = 01 (MEM/WB → ALU) | 2 |
| ForwardC/D (EX/MEM → ID comparator) | 1 |
| Load-use stall | 1 |
| Branch-data stall | 1 |

Nine forwards through the EX/MEM path, seven through MEM/WB, one into the branch comparator. Row 14 of the classification table — the AND R23 instruction — takes ForwardA = 10 from one producer and ForwardB = 01 from another in the same cycle. Both paths active simultaneously on one instruction is the clearest single demonstration that two distinct paths are genuinely required, not one path used twice.

The full nineteen-row table is results/hazard_table_seed16993.txt. Hand analysis predicted 2 stall cycles before the run; both simulators measured 2.

### 9.3 Density suite

| Density | Useful | Cycles (fwd) | Cycles (base) | Stalls | Baseline NOPs | CPI fwd | CPI base | Speedup |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.0 | 48 | 54 | 65 | 2 | 13 | 1.125 | 1.354 | 1.204× |
| 0.1 | 48 | 52 | 62 | 0 | 10 | 1.083 | 1.292 | 1.192× |
| 0.2 | 48 | 53 | 69 | 1 | 17 | 1.104 | 1.438 | 1.302× |
| 0.3 | 48 | 54 | 67 | 2 | 15 | 1.125 | 1.396 | 1.241× |
| 0.4 | 48 | 58 | 75 | 6 | 23 | 1.208 | 1.563 | 1.293× |
| 0.5 | 48 | 56 | 78 | 4 | 26 | 1.167 | 1.625 | 1.393× |
| 0.6 | 48 | 57 | 85 | 5 | 33 | 1.188 | 1.771 | 1.491× |
| 0.7 | 48 | 57 | 95 | 5 | 43 | 1.188 | 1.979 | 1.667× |
| 0.8 | 48 | 62 | 92 | 10 | 40 | 1.292 | 1.917 | 1.484× |
| 0.9 | 48 | 59 | 108 | 7 | 56 | 1.229 | 2.250 | 1.831× |
| 1.0 | 48 | 58 | 112 | 6 | 60 | 1.208 | 2.333 | 1.931× |

Architectural state matches between configurations on all eleven. The shape is the expected one: baseline CPI climbs steeply with density (1.29 → 2.33) while the forwarding configuration stays nearly flat (1.08 → 1.29), so speedup rises from 1.19× to 1.93×. Figures: cpi_vs_density.png, speedup_vs_density.png, cycle_breakdown.png.

The non-monotonicity at 0.8 is real and worth naming: that program has 10 stall cycles against 7 at density 0.9, because target density controls dependence frequency and not the load-versus-ALU mix that determines whether a dependence costs a stall or is forwarded free.

### 9.4 Two independent simulators

Two simulators were written from the same specification without sharing code. Both were run on the assigned routine in both configurations and compared cell by cell across eleven trace columns.

| Configuration | Cycles compared | Columns | Result |
| --- | --- | --- | --- |
| Forwarding | 25 | 11 | PASS |
| Baseline | 47 | 11 | PASS |

Every compared cell agrees. Both also pass all ten test vectors independently. Evidence: results/crossval_hazard.txt, results/crossval_baseline.txt.

### 9.5 Logisim hazard units — and a bug found by cross-validation

Three subcircuits were built in Logisim Evolution: Forwarding_Unit (48 gates), Hazard_Detection_Unit (23 gates) and Branch_Forward_Stall. Each was verified two ways.

Against the Week 1 truth tables. logisim_check.py extracts the gate-level netlist from the .circ file and evaluates it against 3,000 random input vectors per unit. All three: PASS.

Against the golden simulator. logisim_vs_sim.py drives the units with unit_stimulus.csv — the exact per-cycle inputs a datapath would present over 25 cycles of the assigned routine — and compares six outputs per cycle.

Here the as-built circuit failed, on seven of twenty-five cycles:

cycle 4: ForwardC -- circuit says 1, simulator says 0
cycle 5: ForwardC -- circuit says 1, simulator says 0
cycle 6: ForwardD -- circuit says 1, simulator says 0
... 4 more

ForwardA, ForwardB, Stall and BranchStall were correct throughout. Only the branch-comparator forwarding selects diverged, and only on cycles where the instruction in ID was not a branch.

The cause is that the Week 1 equation for ForwardC/ForwardD carries no IsBranch term. It is written in the context of a branch being in ID, and that context was never stated. The circuit implemented the equation literally and therefore asserted the comparator’s forwarding selects on ordinary arithmetic instructions. In a full datapath this would be harmless as long as PCSrc is gated by IsBranch elsewhere — the comparator’s output would be ignored — but it makes the unit’s behaviour undefined in isolation and unverifiable against a trace.

circ_patch.py ANDs IsBranch into both outputs, producing GhanaCore5_HazardUnits_gated.circ, which passes all 25 cycles.

We report both circuits and both results rather than only the working one. The truth-table check passed on the faulty circuit; only cycle-by-cycle comparison against a reference caught it. That is precisely the argument for cycle-level cross-validation over final-state checking, demonstrated on our own work rather than asserted.

### 9.6 Negative test: what the hazard unit prevents

With --no-hdu --allow-mis-forward the mis-forward assertion fires at cycle 6, identifying that EX/MEM holds a load’s address rather than its data. Downgraded to a warning, the program completes and returns plausible wrong numbers:

| Register | Correct | HDU disabled |
| --- | --- | --- |
| R6 | 728 | 519 |
| R8 | 849 | 21 |
| R9 | 1682 | 26 |
| R13 | 833 | 5 |
| R19 | 729 | 520 |
| R23 | 8 | 5 |

No crash, no exception — a yield estimate that is simply wrong. This is the concrete answer to why the interlock exists.

### 9.7 Taken-branch case

The assigned routine’s branch resolves not-taken, so a data overlay that flips the comparison was used to exercise the taken path: 25 cycles, 1 branch taken, 1 flush cycle, 1 branch-data stall. Evidence: results/trace_hazard_branch_taken.txt. Vectors V4 and V5 cover the same pair independently.

## 10. Verification and Validation

| Check | Method | Result |
| --- | --- | --- |
| Functional correctness | 10 hand-derived vectors, expected values written before implementation | 10/10 pass, both simulators |
| Negative test | HDU disabled, mis-forward assertion | fires correctly, both simulators |
| Implementation independence | two simulators, 11 columns, both configurations | PASS |
| Configuration equivalence | word-by-word architectural state | identical, assigned routine + 11 suite programs |
| Hardware logic vs specification | 3,000 random vectors per subcircuit vs Week 1 truth tables | 3/3 PASS |
| Hardware logic vs golden simulator | 25 cycles, 6 outputs | as-built FAIL (§9.5), gated PASS |
| Scheduler equivalence | registers and memory compared after reordering | PASS on all 12 programs |
| Hand prediction vs measurement | classification table predicts 2 stall cycles | both simulators measure 2 |

## 11. Which CPI, and Why It Matters

The baseline’s image contains 43 instructions; the forwarding configuration’s contains 19. Two CPI figures follow, both arithmetically correct, pointing in opposite directions:

| Metric | Baseline | Forwarding |
| --- | --- | --- |
| CPI over useful instructions | 2.4737 | 1.3158 |
| CPI over all image instructions | 1.0930 | 1.3158 |

The second row flatters the baseline badly. NOPs retire one per cycle, so padding drives CPI toward 1.0 while making the program strictly slower. Read in isolation, that row makes forwarding look like a 20% regression when it is in fact 1.88× faster.

CPI over useful instructions is the defensible figure, because it is the denominator the experimental design holds constant. This is Hennessy and Patterson’s Chapter 1 caution appearing directly in our own data. We report both because reporting only one would conceal that the trap exists.

## 12. Analytical CPI Model

matlab/stall_cpi_model.m models CPI as a function of hazard density and validates it against the eleven measured points in cpi_sweep.csv.

CPI_fwd  = 1 + k/N + f_stall                      (k = 4 fill cycles, N = 48)
CPI_base = 1 + k/N + NOPs/N

The forwarding model is well behaved because its penalties are local and countable: one cycle per surviving stall. The baseline model is harder, because the padder’s requirement — a consumer at slot c must satisfy c ≥ p + 3 — is measured in padded positions, so NOPs inserted for an early dependence separate later instructions for free and one NOP can satisfy several dependences at once. The classification table notes this explicitly: per-row base costs sum to more than the 24 NOPs actually inserted, and the padder total is the figure to quote.

## 13. Innovation Component: Forwarding-Aware Scheduling

### 13.1 Motivation

Full forwarding leaves exactly two stall classes, and both are distance-1 problems. A load’s data does not exist until the end of MEM; a branch resolves in ID before a producer in EX has finished. Neither is a hardware limit in the interesting sense: move a legal independent instruction into the gap, the dependence becomes distance-2, and an existing forwarding path covers it. No new hardware.

### 13.2 Method

Parse; build a per-basic-block dependence graph over RAW, WAR, WAW and memory ordering; locate distance-1 stall pairs; search both directions for a legal filler; verify; emit. Memory operations are conservatively ordered whenever one is a store — no address disambiguation is attempted. This may forgo a legal move; it cannot admit an illegal one. Semantic equivalence is verified after reordering by comparing registers and memory, not assumed from the move logic.

### 13.3 Results

Assigned routine: 25 → 24 cycles, load-use stalls 1 → 0, CPI 1.3158 → 1.2632, equivalence PASS. One cycle is the ceiling here, because the routine contains exactly one load-use hazard.

Across the eleven-program density suite:

| Density | Cycles | Scheduled | Stalls | Removed | Equivalence |
| --- | --- | --- | --- | --- | --- |
| 0.0 | 54 | 52 | 2 | 2 | PASS |
| 0.1 | 52 | 52 | 0 | 0 | PASS |
| 0.2 | 53 | 52 | 1 | 1 | PASS |
| 0.3 | 54 | 52 | 2 | 2 | PASS |
| 0.4 | 58 | 53 | 6 | 5 | PASS |
| 0.5 | 56 | 52 | 4 | 4 | PASS |
| 0.6 | 57 | 53 | 5 | 4 | PASS |
| 0.7 | 57 | 53 | 5 | 4 | PASS |
| 0.8 | 62 | 53 | 10 | 9 | PASS |
| 0.9 | 59 | 55 | 7 | 4 | PASS |
| 1.0 | 58 | 53 | 6 | 5 | PASS |

620 → 580 cycles, 6.5% saved. 40 of 48 stalls removed (83%). Semantic equivalence on every program.

The result the suite establishes that one program could not: scheduled cycle counts converge on 52–55 almost independently of hazard density, while unscheduled counts spread from 52 to 62. Instruction ordering, not dependence frequency, accounts for most of the residual stall cost once forwarding is present.

The eight stalls not removed are genuine. At density 0.9 only 4 of 7 come out, because a filler must be independent of both the producer and the consumer and live in the same basic block; in dependence-dense regions no such instruction exists.

## 14. Limitations and Threats to Validity

The Logisim datapath is not built. This is the honest headline limitation. The three hazard units exist, are verified against the truth tables and against the golden simulator on every cycle — but there is no PC, instruction ROM, pipeline register, register file, ALU or top-level circuit, so the circuit does not execute the routine end to end. The team did not carry a Project-2 datapath forward and would have had to build one from scratch. What is established is the weaker but substantive claim that the hazard-control logic reproduces every forwarding, stall and comparator decision the simulator takes, driven by exactly the stimulus a datapath would present. What is not established is that a complete circuit runs the program.

Cycles are not time. Forwarding adds multiplexers to the EX-stage critical path and could reduce achievable clock frequency. No timing analysis was performed, so 1.880× is an upper bound on a real speedup, not a measurement of one.

One kernel, one domain. Twelve programs, all arithmetic-dense straight-line code with at most one branch. Nothing here shows the result generalises to branch-dense or memory-bound code.

Density does not control the load/ALU mix. Target density sets dependence frequency, not whether a dependence is a load-use (costs a stall) or ALU-to-ALU (forwarded free). This is why density 0.8 has more stalls than 0.9 (§9.3), and it weakens density as a single explanatory variable.

The scheduler is greedy and local. First legal improving move within a basic block, no cross-block motion, conservative memory model.

Seed is provisional. 16993 was drawn on 19 August 2026 and documented in the proposal; the instructor-assigned seed had not been issued. On receipt, rebuild.py --seed <official> re-derives every number.

## 15. Conclusions

Forwarding and hazard detection reduce the COCOBOD routine from 47 to 25 cycles, 1.880×, with identical results. Seventeen of nineteen dependence events resolve at zero cost. The two that do not are ordering artefacts, and scheduling removes 40 of 48 such stalls across the density suite with no hardware change.

Two findings we did not expect. First, once forwarding is present, scheduled cycle counts are nearly independent of hazard density — ordering dominates the residual, not dependence frequency. Second, our Logisim units passed a 3,000-vector truth-table check while still being wrong, and only cycle-by-cycle comparison against a reference implementation caught the missing IsBranch gate. Verifying against the specification and verifying against a reference are not the same activity, and we would not have learned that from a passing result.

## 16. Reproducibility Statement

python3 rebuild.py --seed 16993

Requires gcc and Python 3.10 or later. Regenerates the program, both configurations, the scheduled variant, the density suite, the Logisim ROM images and every file in results/, and re-runs every check.

| Artefact | Command |
| --- | --- |
| Test vectors | python3 tests/hazard_test_vectors.py, tests/extra_test_vectors.py |
| Simulator cross-validation | python3 tools/crossval_sims.py |
| Logisim netlist check | python3 tools/logisim_check.py |
| Logisim vs simulator | python3 tools/logisim_vs_sim.py |
| Scheduler | python3 tools/scheduler.py |
| Density sweep | python3 tools/sweep.py |
| MATLAB figures | from matlab/: >> stall_cpi_model |

## 17. Individual Contributions

| Member | Artefacts personally authored | Report sections | Commits |
| --- | --- | --- | --- |
| Ishaan Bhardwaj | gen_cocobod_instrs.py, gen_density_suite.py, sweep.py, plot_cpi.py, stall_cpi_model.m, rebuild.py | §8, §11, §12 | [hashes] |
| Nyavor Cyril | Week 1 design document and truth tables, GhanaCore5_HazardUnits.circ, gen_logisim_roms.py, circ_patch.py, logisim_check.py, logisim_vs_sim.py | §5, §9.5 | [hashes] |
| Prince Philips | hazard_pipeline_sim.c, second implementation (sim/), crossval_sims.py | §6, §9.4, §9.6 | [hashes] |
| Samia Soleimani | hazard_test_vectors.py, extra_test_vectors.py, crossval.py, scheduler.py, STATUS.md, README | §7, §10, §13, §14 | [hashes] |

Consistent with the individual contribution forms submitted separately.

## 18. AI-Use Summary

AI assistance (Claude, Anthropic) was used substantively to draft simulator code, analysis and scheduling scripts, verification tooling and documentation. It was not used for the Week 1 design document, the hand-derived test vectors, or the Logisim circuit construction. Every AI-assisted artefact was verified by the member who owns it. Full declaration with dates, components and verification method: WEEK4_SUBMISSION/07_ai_use/.

## 19. Acknowledgements

[Optional.]

## 20. References

- J. L. Hennessy and D. A. Patterson, Computer Architecture: A Quantitative Approach, 6th ed. Cambridge, MA: Morgan Kaufmann, 2019.

- [Week 1 reviewed paper 1 — full IEEE citation with DOI.]

- [Week 1 reviewed paper 2 — full IEEE citation with DOI.]

## Appendices

|  | Content | File |
| --- | --- | --- |
| A | Hazard classification table, 19 rows | results/hazard_table_seed16993.txt |
| B | Test vectors, both simulators | results/test_vectors.txt |
| C | Per-cycle traces | results/percycle_baseline.csv, percycle_hazard.csv |
| D | Simulator cross-validation | results/crossval_baseline.txt, crossval_hazard.txt |
| E | Logisim verification | results/logisim_check_*.txt, logisim_vs_sim_*.txt |
| F | Density sweep and scheduler | results/cpi_sweep.csv, scheduler_suite_report.txt |
| G | Mis-forward negative test | results/misforward_assertion.txt |